UNIT 2 – NETWORK MODELS AND PROTOCOLS
AND DATA COMMUNICATION

PRN:2125UCSM1097
MINI-PROJECT REPORT

Protocol Analysis with Wireshark

| Student Name | PRATAP AHER (2125UCSM1097) |
| --- | --- |
| Project | Protocol Analysis with Wireshark |
| Software | Wireshark (v4.x) & Mozilla Firefox |
| Environment | Kali Linux (eth0) |
| Target Server | neverssl.com (34.223.124.45:80) |
| GitHub Repository | Protocol-Analysis-with-Wireshark |
| GitHub Link | https://github.com/pratapaher/PRATAP-Protocol-Analysis-with-Wireshark-NPC.git |

Submitted as part of Unit 2 practical / mini-project

# 1. Objective

To capture and analyze live network packets using Wireshark on Kali Linux, examine the TCP three-way handshake (SYN, SYN-ACK, ACK), and observe the subsequent HTTP GET request and HTTP 200 OK response.

# 2. Software and Requirements

| Device / Software | Quantity | Purpose |
| --- | --- | --- |
| Kali Linux | 1 | Operating system and network platform |
| Wireshark (v4.x) | 1 | Network packet analyzer and deep inspection |
| Mozilla Firefox | 1 | Web browser to generate HTTP traffic |
| Ethernet Interface (eth0) | 1 | Active network link for packet capture |
| Target (neverssl.com) | 1 | Remote HTTP web server for cleartext GET |

# 3. Network Setup and Environment

Packet capture was executed on Kali Linux over interface eth0. Cleartext HTTP web traffic was generated by requesting http://neverssl.com (IP: 34.223.124.45) on TCP port 80.

Figure 1: TCP Three-Way Handshake & HTTP GET Packet (tcp.stream eq 7)

# 4. Capture and Analysis Procedure

# The following systematic procedure was executed to capture and inspect the network traffic:

Open Wireshark in Kali Linux with administrative privileges and select eth0 interface.

Start live packet capture to record incoming and outgoing frames.

Launch Mozilla Firefox and navigate to http://neverssl.com.

Wait for the webpage to fully load, then stop the packet capture in Wireshark.

Apply the display filter 'http' to locate the HTTP web traffic.

Inspect the HTTP GET request and examine the TCP stream (tcp.stream eq 7).

# 5. Transport Layer Analysis

The transport-layer protocol used is TCP. The client initiated communication from ephemeral port 34078, connecting to web server destination port 80. The three-way handshake established a reliable connection with synchronized sequence numbers prior to application-layer HTTP transfer.

| Packet | Frame No. | Source (IP:Port) | Destination (IP:Port) | TCP Flags / Status |
| --- | --- | --- | --- | --- |
| SYN | 87 | 192.168.17.130:34078 | 34.223.124.45:80 | [SYN] Seq=0 |
| SYN-ACK | 103 | 34.223.124.45:80 | 192.168.17.130:34078 | [SYN, ACK] Ack=1 |
| ACK | 104 | 192.168.17.130:34078 | 34.223.124.45:80 | [ACK] Seq=1, Ack=1 |
| HTTP GET | 105 | 192.168.17.130:34078 | 34.223.124.45:80 | GET / HTTP/1.1 |
| HTTP 200 OK | 108 | 34.223.124.45:80 | 192.168.17.130:34078 | 200 OK (text/html) |

# 6. HTTP Traffic Verification

The HTTP traffic verification was performed by applying the display filter 'http'. Wireshark isolated the application-layer request and server response:

Observed HTTP packets:

Packet 105: GET / HTTP/1.1 (Client Request to neverssl.com)

Packet 108: HTTP/1.1 200 OK (Server Response containing HTML body)

Figure 2: HTTP GET Request & HTTP 200 OK Response (http filter)

# 7. Observation Summary

| Client / Source | Server / Destination | Protocol Event | Status |
| --- | --- | --- | --- |
| Client (Kali) | Server (34.223.124.45) | TCP 3-Way Handshake | ESTABLISHED |
| Client (Kali) | Server (34.223.124.45) | HTTP GET / HTTP 200 OK | SUCCESSFUL |

# 8. Result

Network traffic was successfully captured and analyzed using Wireshark on Kali Linux. The TCP three-way handshake (SYN, SYN-ACK, ACK) and subsequent HTTP GET request and 200 OK response were identified successfully. All source and destination IP addresses and TCP port numbers were recorded from the capture.

# 9. Conclusion

This practical demonstrated how Wireshark can be used to inspect network protocols at the packet level. The analysis confirmed that TCP creates a reliable virtual circuit using sequence and acknowledgment numbers before application-layer HTTP traffic flows. Wireshark provides valuable visibility into OSI Layer 3 (Network) and Layer 4 (Transport) operations.

# 10. Submission Evidence Checklist

✓ Wireshark packet capture file: protocol_analysis.pcapng

✓ TCP Handshake Screenshot: screenshots/Figure_1_TCP_Handshake.png

✓ HTTP Traffic Screenshot: screenshots/Figure_2_HTTP_Traffic.png

✓ GitHub Repository: https://github.com/ShubhamShinde148/Protocol-Analysis-with-Wireshark